# Lynx

# Software DATV Receiver

Web UI & Configuration Reference

G8YTZ / EI3IOB — July 2026 — Version 1.1

This is a quick reference for what's actually on screen, not a design or installation document — see the separate Installation Guide for getting a receiver running from a blank SD card, and the Diversity Reception manual for how the combiner and recovery logic actually work internally. This one just answers “what does this button/field do.”

The Web UI has three pages: the main receiver page (/), Diagnostics (/diagnostics), and Configuration (/config) — reachable from each other via the header links on the main page.

# 1. Main receiver page

## 1.1 Header

Top-right, alongside the receiver title: a Picotuner online/offline indicator, the current mode badge (IDLE / RF / STREAM), a small mpv: N badge linking to Diagnostics (the number is the mpv restart count for this session), and two buttons — Diagnostics and Config — linking directly to the other two pages.

## 1.2 Tuner Rx 1 / Tuner Rx 2 (Diversity)

Live status for each Picotuner: lock state, callsign, MER, margin, frequency, symbol rate, modcod, codec, signal level (dBm), and programme name. Rx 2 only appears when diversity mode is enabled and shows tuner B's own, independent readings — this is raw per-tuner status, not the combined output.

Signal level shows a genuine, calibrated dBm figure on current Picotuner firmware (ptwh0v3k+ or later) for both tuners; older firmware falls back to a rougher approximation instead. Programme name for tuner B specifically only became available with that same firmware update — on older firmware it stays blank, which is expected, not a fault.

## 1.3 RF Reception (Picotuner)

Presets — saved memories, either an RF frequency/symbol-rate combination or a saved stream; click one to tune or start playing it immediately. Stream-type memories are marked with a 📶 icon in the list to distinguish them from RF ones at a glance.

Manual Tune — frequency (kHz) and symbol rate (kS/s), an LNB selector (for satellite work — sets the local-oscillator offset so the frequency field represents the true downlink frequency rather than the IF), and a plug selector:

| Option | Meaning |
| --- | --- |
| Plug A (top) | Single-receiver mode, tuner A only |
| Plug B (bottom) | Single-receiver mode, tuner B only |
| Diversity (A+B) | Both tuners combined via the diversity combiner |

Two small buttons next to Tune: 💾 saves the current frequency/symbol rate as a new preset; 🏠 sets it as the boot-time default (see below).

Boot default. On startup, Lynx normally resumes whatever it was last tuned to. If there's no previous state to resume (e.g. a fresh install, or the state file was cleared), it falls back to this saved default instead. There isn't always one set — the small note under the Tune row shows the current default, if any.

## 1.4 Network Streams

Live BATC stream list (refreshable), plus a manual entry field for any other RTMP, SRT, UDP, or RTSP URL. Selecting a stream switches Lynx into stream mode, playing that URL directly rather than the RF path.

Every stream in the list, and the manual URL field, now has its own 💾 button — saves that stream as a named memory, appearing in the same Presets list as RF frequencies (§1.3), and usable from the M5Dial front panel the same way an RF memory is.

## 1.5 Control

Stop — stops mpv entirely; shows the idle/cover screen. Does not affect RF lock itself, only what's being displayed.

Reboot — reboots the whole Raspberry Pi (not just the Lynx software), via sudo reboot. Asks for confirmation first, since this is disruptive — the receiver and this page will be briefly unreachable while it comes back up.

Volume — 100% is unity gain (0dB), the correct setting for sources already mastered to broadcast-normal loudness; it is not “maximum volume” in the usual consumer-device sense. A separate field below sets the default volume applied on boot.

API Quick Reference — the handful of endpoints most likely to be scripted against directly (tune, stream, volume, stop, status), with a link through to the full, interactive API documentation at /docs.

## 1.6 Current Config

A read-only summary of the site/Picotuner/diversity configuration currently in effect, with an Edit link through to the full Configuration page (§3).

# 2. Diagnostics page

Auto-refreshing (every 5 seconds) record of every mpv start, stop, and restart, plus related events (modcod changes, Picotuner connectivity, drift-correction actions) — all on one shared timeline so cause and effect can be read directly against each other, rather than cross-referencing separate logs by hand.

Total mpv events — restart count for the current session.

By reason — the same total, broken down by why each restart happened.

Recent events — the last 200 events, newest first, with an exact timestamp and a plain-text detail for each.

### Event categories

| Category | Meaning |
| --- | --- |
| RF: lock confirmed, mpv started | A genuine, sustained lock was detected and mpv started |
| RF: lock lost, mpv stopped | Lock was sustained-lost and mpv was stopped |
| Modcod changed | The Picotuner reported a different modcod on tuner A or B |
| Decoder: HEVC reference errors | mpv's decoder logged a burst of HEVC reference errors |
| Decoder: playback delay grew | mpv fell behind live by more than the configured threshold |
| Decoder: restarts not helping, backed off | Repeated decoder-triggered restarts didn't resolve the issue - backed off for a cooldown |
| Drift correction: drop-buffers resync | Drift exceeded the speed-nudge-only threshold, forcing an immediate resync |
| Drift correction: repeated resyncs not helping, backed off | Same idea as the decoder breaker above, for drift resyncs |
| Hard freeze detected - immediate mpv restart | Playback stopped advancing entirely - restarted immediately rather than waiting on the slower playback-delay trigger |
| Hard freeze detected - restart breaker active, suppressed | A hard freeze happened while the restart breaker was tripped and the combiner's own output wasn't yet confirmed clean enough to retry early |
| Hard freeze restarts not helping, backed off | Too many hard-freeze restarts in quick succession - backing off (see §3.3, Hard-Freeze Recovery) |
| Picotuner went offline / back online | Connectivity to the Picotuner itself was lost or restored |
| Stream render slow | A stream switch didn't confirm mpv was genuinely rendering within the initial timeout - a second, more patient attempt was made before giving up |
| Stream render not confirmed | A stream switch never confirmed genuine rendering even after a second attempt - the transition cover was deliberately left up rather than risk exposing the desktop underneath |

A handful of newer, more specific diagnostic categories currently show their raw internal name rather than a friendly label on this page - a small, known gap, not otherwise affecting anything.

# 3. Configuration page

Three columns, grouped by topic. Every section has its own Save button and its own independent status message - saving one section never touches any other.

## 3.1 Site Information

Receiver name, callsign, location, and locator - used for on-screen display and, where relevant, in outgoing notifications (QRZ log entries, Slack messages).

## 3.2 Picotuner Network Settings

The Picotuner's IP address and its five ports (status, and command/TS pairs for tuners A and B). Requires a Lynx restart - these are read once at startup by background monitoring threads, not re-read live.

## 3.3 Diversity Source Switching

MER hysteresis — Dwell time and MER margin: when both tuners are clean, the combiner sticks with whichever source is currently preferred rather than re-deciding every segment. The preferred source only changes once the other tuner's MER has been consistently better by at least the margin, for at least the dwell time - not on a single momentary blip. Requires a combiner restart (the diversity combiner is a separate process; these are passed to it as startup arguments).

Hard-Freeze Recovery (stacked below Slack in column 2) — governs how the hard-freeze restart breaker behaves. Applies immediately, no restart needed.

| Field | Meaning |
| --- | --- |
| Enabled | Turns the breaker on/off entirely. Disabled means every hard freeze restarts immediately, with no cooldown logic at all. |
| Trip after this many restarts / within this many secs | How many hard-freeze restarts in how short a window counts as “this isn't a one-off” and engages the breaker. |
| Max cooldown | The upper bound on how long the breaker suppresses restarts for, if conditions never clear. |
| Required clean time | Once tripped, the breaker checks the combiner's own output before each retry - this many seconds of genuinely clean data (no gaps or corruption) is enough to retry early, rather than waiting out the full cooldown. |
| Min. seconds between early retries | A floor on how often an early, condition-based retry can happen, even if the combiner looks clean throughout - stops a flapping clean/dirty signal from becoming a tight retry loop. |

## 3.4 QRZ.com Logbook

Logs a QSO automatically once a station has been locked and settled for the configured time. The API key determines which QRZ account/logbook entries are posted to.

| Field | Meaning |
| --- | --- |
| Enabled | Turns logging on/off |
| API key | Your QRZ Logbook API key |
| Settle time | Delay after lock before logging, so the callsign has time to decode |
| Suppress | Don't log the same callsign again within this many minutes |

## 3.5 Bitfocus Companion

Two independent HTTP triggers, fired on lock and unlock respectively, each with its own settle time - typically used to switch a vision mixer or similar to/from this receiver's output automatically.

An optional second output, further down the same card, mirrors the same lock/unlock state on a GPIO pin instead of (or alongside) the webhooks - for relay-based input switching hardware. It deliberately reuses the same two settle times above rather than having its own.

| Field | Meaning |
| --- | --- |
| Lock URL / Unlock URL | The two webhook endpoints |
| Settle time (each) | Delay after lock/unlock before firing that webhook |
| Also mirror on a GPIO pin | Enables the relay output |
| Physical pin / Polarity | Which pin, and whether “locked” means that pin going high or low |

## 3.6 Slack

Sends a message on lock, after its own settle time, built from a free-text template with placeholders substituted in:

{site_callsign} {site_callsign_lower} {rx_callsign} {mer} {margin} {modcod} {frequency}

## 3.7 GPIO Tx On/Off

Controls a GPIO pin intended for switching a transmitter/PA on and off based on reception - e.g. only powering up repeater transmit hardware while a signal is actually being received.

| Field | Meaning |
| --- | --- |
| Physical pin / Polarity | Which pin, and which level means “on” |
| Power-up settle (s) | Delay after lock before switching on |
| Power-down settle (s) | Delay after unlock before switching off - 0 means never auto power-down once triggered on |
| Weekday / weekend schedule | An optional daily time window where the pin is forced on immediately, overriding the settle-based logic above for that window. Leave “No schedule” ticked for pure signal-driven behaviour with no schedule at all. |

Outside a configured window (or with no schedule set for that day type), the normal power-up/power-down settle times above apply continuously.

# Document Version History

| Version | Changes |
| --- | --- |
| 1.0 | Initial reference, covering the main receiver page, Diagnostics, and Configuration as they stood at the time. |
| 1.1 | Added dBm and programme-name fields to tuner status (§1.2); updated Presets to reflect the unified RF/stream memory system and the new save-as-memory button on streams (§1.3, §1.4); added two new diagnostic event categories from the stream render-confirmation fix (§2). §3.2 unchanged - Picotuner network settings are genuinely editable from this page, as originally documented. |